# PR301	Mahindra Ecole Centrale	2018-19/II

Experiment  5 	Interrupts, Timers, DAC and ADC

The objective of this experiment is to get familiar with the use of the following features of a microcontroller: Interrupt, Timer, DAC and ADC. A triangular waveform having frequency adjustable through a Timer will be generated at the output of a DAC through a programme triggered by an Interrupt from a Timer. Another Timer will then be used to sample the triangular waveform and an ADC used to convert these samples back to Digital. Configurable interconnections among the microcontroller peripherals will be utilised to set up the whole system.

Part A: Generation of Triangular Waveform by DAC triggered by a Timer

A triangular waveform can be generated by a DAC by applying to the input of the DAC a repetitive sequence of numbers that increase linearly from zero up to a prescribed maximum value and then decreasing linearly back to zero. As the microcontroller has a built-in reference voltage VREF+ = 3.3V, the analog output voltage VA is given in terms of the digital 12-bit input ND by the relationship VA = 3.3ND / 4096. So if ND is incremented in 16 steps of 256 (=100H) each from an initial value 000H, and then decremented in 16 steps of the same step size, VA will go up linearly from 0V to 3.3V in 16 steps, and then go down linearly to 0V in 16 steps. This cycle can be repeated endlessly to generate a triangular wave having a peak value 3.3V. The time period of the triangular wave will then be 32 times the time taken for each step in the DAC input sequence, which is given by the time interval between successive Update Event (UEV) outputs from the Timer configured in the basic Time Base mode. The Frequency of UEV output is given by:

fUEV = fPSC /{(PSC + 1) (ARR + 1) (RCR + 1)},

where PSC, ARR and RCR denote the values stored in the Pre-Scaler Counter, Auto-Reload Register and Repeat Counter Register respectively.

- Set up the Timer TIM3 in the basic Time Base mode with an 8MHz Clock input and with PSC = 79 and RCR = 0 to generate Interrupts at time intervals TSTEP = 10(ARR + 1) s.

- Write an ISR to generate a periodic DAC input going up from 0 to 4096 in 16 equal steps, and then coming down from 4096 to 0 in 16 equal steps again, with step size of 256. [Ref. Sections 16.1 – 16.5 of RM0316]

- Display, using the built-in Logic Analyser, the DAC output as a Curve on your laptop screen for ARR = 9, 19 and 39. Note how the shape of the displayed waveform is affected by the time interval between consecutive Interrupts, as decided by the value of ARR. How would you estimate the time taken by the ISR to generate successive points on the DAC output?

Part B: Sampling the Triangular Waveform by ADC triggered by a Timer

- Set the DAC output for a time period of 6.4ms (ARR = 19), and connect it to an ADC input. Use TIM1 to trigger the ADC at intervals of 0.8ms by suitably choosing the Timer parameters, using the internal connection between TIM1 channel outputs and ADC trigger inputs, as given in the interconnections among peripherals. [Ref. Sections 15.3.13 and 15.3.18 of RM0316]

- Tabulate the values of the ADC output in Hex format for two complete cycles of the triangular waveform, along with the values given by the triangular DAC output waveform. Compare the corresponding values and comment on the error.

- Repeat step B.2 for ARR = 9 and 39. Comment on the effect on the error.

STM Cube Configuration Steps for DAC with TIM3

- Select RCC from the pinout pane, and choose High Speed Clock (HSE): Crystal/Ceramic Resonator.

- Select TIM3 and TIM1 from the pinout pane, and choose: Clock Source: Internal Clock.

- Select DAC1 from the pinout pane: Select OUT1 Configuration.

- Select ADC1 from the pinout pane, and select N1: Single-ended.

- From the Clock Configuration Set the Timer Clock to 8 MHz.

- Set the following parameters for TIM3 from the configuration pane:

Pre-scalar = 79, Counter Mode UP, Counter Period (ARR) = 19.

    - NVIC Settings: Check the TIM3 Global Interrupt enable.

- Set the following parameters for DAC1 from the configuration pane:

    - Parameter Settings: Check the output buffer to enable.

STM Cube Configuration Steps for ADC with TIM1

- Set the following parameters for TIM1 from the configuration pane:

Parameter Settings: Pre-scalar = 79, Counter Mode UP, Counter Period (ARR) = 79.

Trigger Event Selection TRGO to Update Event

- Set the following parameters for ADC1 under ADC Regular Conversion Mode:

      - External Trigger Conversion Source: Timer 1 Trigger out event

      - NVIC Settings: Check the ADC1 and ADC2 Interrupts enable

Building the complete Project to generate the Keil code for ARM-MDK5

- Insert parameter values as required.

  - Define Private Variables for DAC output, ADC output.

  - Under user code begin 2 of Main function
HAL_TIM_Base_Start_IT(&htim3);  // This will create the events periodically

  - On top of the Main function i.e under User code Begin 0.

void HAL_TIM_PeriodElapsedCallback(TIM_HandleTypeDef *htim)

{

if(htim->Instance==TIM3)

{

// Write your Triangular wave Generation code here by using appropriate variables

}

HAL_DAC_Start(&hdac, DAC_CHANNEL);

HAL_DAC_SetValue(&hdac, DAC_CHANNEL,DAC_ALIGNMENT, dac_value_variable);

dac_output_variable=(3300* dac_value_variable)/4096;

}

  - Then compile and flash the program in the microcontroller and run in debug mode to check the wave form using logic analyser.

  - Under user code begin 2 of Main function
HAL_TIM_Base_Start(&htim1);  // This will trigger the ADC

HAL_ADC_Start_IT(&hadc);    // Starting the ADC

void HAL_ADC_ConvCpltCallback(ADC_HandleTypeDef* hadc)

{

if(hadc->Instance == ADC1)

{

HAL_ADC_PollForConversion(hadc,100);

adc_output=HAL_ADC_GetValue(hadc);

}

}

- Now compile the both the codes and connect the header wire between the DAC output and ADC output to generate the ADC output waveform. Compare the two waveforms and comment.